TECHNICAL RESEARCH & IMPLEMENTATION BLUEPRINT

Vision-Based Autonomous Navigation for
Unmanned Ground Vehicle in Outdoor Environments

Problem Statement ID: 26126

Organization: Bharat Electronics Limited (BEL)

Proposed hybrid stack: NVIDIA Isaac Sim + ROS 2 + RTAB-Map + GPU Elevation/Traversability + Nav2 + Vision AI

Prepared as a research, architecture, implementation, simulation, validation and deployment guide

# Document Control

| Item | Value |
| --- | --- |
| Problem Statement | 26126 |
| Title | Vision Based Autonomous Navigation for Unmanned Ground Vehicle for Outdoor environment |
| Organization | Bharat Electronics Limited |
| Document purpose | Research and implementation blueprint for a simulation-first autonomous UGV solution |
| Primary test strategy | Software-in-the-loop / simulation before physical hardware |
| Recommended simulator | NVIDIA Isaac Sim 6.x |
| Recommended middleware | ROS 2 Jazzy |
| Primary localization | RTAB-Map |
| Primary navigation | Nav2 |
| Terrain representation | GPU-accelerated elevation map + traversability costmap |
| Document status | Engineering proposal / research baseline |
| Information cutoff | 4 September 2026 |

## Important Scope Clarification

No software stack can honestly guarantee 100% real-world perception or navigation accuracy. Outdoor autonomy is affected by lighting, weather, terrain ambiguity, sensor contamination, dynamic obstacles, localization failure and hardware faults. This document therefore defines the engineering target as high-probability, collision-free mission completion with explicit uncertainty handling and a fail-safe stop policy. The proposed validation target is to demonstrate repeatable collision-free operation across a large randomized simulation suite and then reproduce the same software architecture on hardware.

Engineering note: For a competition, claim measurable success metrics such as collision-free completion rate, localization error, path efficiency and recovery rate rather than '100% accuracy'.

# Contents

1. Executive Summary

2. Problem Statement Interpretation

3. Requirements and Success Criteria

4. Research Findings

5. Existing Software and Open-Source Landscape

6. Technology Selection Matrix

7. Proposed Hybrid Architecture

8. Detailed Perception System

9. Visual Localization and SLAM

10. Elevation Mapping and Traversability

11. Navigation, Planning and Control

12. Dynamic Obstacle Avoidance

13. Safety Supervisor and Fail-Safe Design

14. Simulation Strategy

15. Isaac Sim Environment Design

16. ROS 2 Integration Architecture

17. Software Repository Map

18. Data and AI Strategy

19. Testing and Validation Plan

20. Evaluation Metrics

21. Machine and Hardware Requirements

22. Installation and Development Plan

23. Implementation Roadmap

24. Failure Modes and Mitigations

25. Security, Licensing and Open-Source Compliance

26. Deployment Path to a Physical UGV

27. Recommended Final Stack

28. Competition Demonstration Plan

29. Conclusion

Appendix A — Repository and Software Reference

Appendix B — Example ROS 2 Topic Contract

Appendix C — Example Cost Function

Appendix D — Test Scenario Catalog

Appendix E — References

# 1. Executive Summary

Problem Statement 26126 asks for an autonomous outdoor UGV that can operate without dependable GPS, using onboard computer vision as the primary sensing modality. The required capabilities are: (1) identify safe traversable paths and hazards, (2) estimate position and orientation visually, and (3) dynamically avoid obstacles while reaching a destination.

The lowest-effort credible solution is not to build a complete autonomy stack from scratch. Instead, the proposed system combines mature robotics frameworks and open-source research components, with custom engineering concentrated where the BEL problem is genuinely specific: camera-derived terrain understanding, semantic/geometry fusion, safety logic, integration, calibration and evaluation.

| Function | Recommended technology | Why |
| --- | --- | --- |
| Simulation | NVIDIA Isaac Sim 6.x | Photorealistic sensor simulation, physics, ROS 2 bridge and synthetic data |
| Middleware | ROS 2 Jazzy | Standard interface between sensors, localization, perception, mapping and navigation |
| Localization/SLAM | RTAB-Map ROS 2 | Supports stereo/RGB-D and has Nav2 integration examples |
| Terrain mapping | elevation_mapping_cupy | GPU-accelerated elevation map, semantic layers and traversability filtering |
| Navigation | Nav2 | Mature ROS 2 navigation framework with costmaps, planners, controllers and recovery |
| Local control | Nav2 MPPI controller | Predictive local control and obstacle-aware trajectory selection |
| Perception | YOLO/segmentation + depth | Semantic hazards plus geometry |
| Point-cloud processing | Open3D / PCL where needed | Efficient 3D processing and debugging |
| Safety | Custom ROS 2 supervisor | Deterministic handling of stale sensors, localization loss and emergency stop |

The system should be developed simulation-first. A virtual UGV in Isaac Sim publishes camera, depth, IMU and odometry data through ROS 2. RTAB-Map estimates pose; vision models identify semantic terrain and hazards; an elevation/traversability layer turns geometry and semantics into navigation costs; Nav2 plans and controls the vehicle; and a small safety supervisor can override movement whenever confidence or sensor health falls below defined limits.

# 2. Problem Statement Interpretation

## 2.1 What BEL is really asking for

Perception: distinguish traversable ground from hazards such as rocks, ditches and trees.

Localization: estimate vehicle pose without relying on GPS.

Navigation: convert the environment representation into a safe route toward a destination.

Collision avoidance: react to obstacles that appear after the initial route has been planned.

Outdoor robustness: tolerate changing illumination and unstructured terrain.

Functional demonstration: successfully navigate from Point A to Point B without collision.

## 2.2 The problem as a continuous loop

CAMERA / IMU
    │
    ├──► Perception ───────► "What is around me?"
    │
    ├──► Visual SLAM ──────► "Where am I?"
    │
    └──► Depth / Geometry ─► "What terrain is drivable?"
                  │
                  ▼
          Traversability Costmap
                  │
                  ▼
              Nav2 Planner
                  │
                  ▼
             Local Controller
                  │
                  ▼
               /cmd_vel
                  │
                  ▼
                 UGV
                  │
                  └──────────────► CAMERA (repeat)

The critical architectural principle is separation of concerns. Neural networks should primarily handle uncertain perception, while deterministic robotics software should handle coordinate transforms, mapping, path planning, control and safety.

# 3. Requirements and Success Criteria

## 3.1 Functional requirements

| ID | Requirement | Acceptance evidence |
| --- | --- | --- |
| FR-01 | Detect or estimate traversable terrain | Traversability layer visible in RViz and consumed by planner |
| FR-02 | Detect hazardous objects/regions | Annotated camera stream + obstacle costmap |
| FR-03 | Estimate pose without GPS | RTAB-Map trajectory compared with simulator ground truth |
| FR-04 | Plan Point A → Point B | Nav2 reaches goal repeatedly |
| FR-05 | Avoid newly appearing obstacles | Dynamic obstacle causes local replan or stop |
| FR-06 | Handle uncertainty | Low confidence causes reduced speed or stop |
| FR-07 | Recover from temporary blockage | Planner/controller performs recovery or safely stops |
| FR-08 | Run in real time | Perception, mapping and control meet declared latency budgets |

## 3.2 Non-functional requirements

Modular ROS 2 nodes with explicit topic and frame contracts.

Reproducible simulation using version-pinned containers or environment manifests.

Logging of all sensor, pose, planner and safety events.

Deterministic emergency-stop behavior independent of AI confidence.

Hardware-agnostic sensor abstraction so the simulation stack can later consume real camera data.

Open-source license review before redistribution or commercial deployment.

## 3.3 Proposed engineering targets

| Metric | Prototype target | Competition target |
| --- | --- | --- |
| Collision-free completion | ≥ 95% randomized missions | ≥ 99% on defined evaluation suite |
| Goal completion | ≥ 97% | ≥ 99% |
| Emergency stop response | < 200 ms software path | < 100 ms where hardware permits |
| Localization drift | < 2% of traveled distance in controlled scenes | < 1–2% after tuning |
| Perception latency | ≤ 100 ms | ≤ 60 ms preferred |
| Control loop | ≥ 20 Hz | ≥ 30 Hz preferred |
| Planner update | ≥ 5 Hz | ≥ 10 Hz preferred |

Engineering note: These are proposed project targets, not guaranteed specifications of the cited software.

# 4. Research Findings

The research review indicates that the project does not require inventing SLAM or navigation algorithms. Mature components already exist for visual SLAM, ROS 2 navigation, elevation mapping and simulation. The research gap is integration under the exact constraint of vision-first outdoor navigation.

## 4.1 Most relevant existing stack

LARIAD/Offroad-Nav is the closest conceptual match discovered. Its repository describes an open-source LiDAR and monocular off-road autonomous navigation stack for a UGV, with Isaac Sim simulation, elevation mapping, a monocular Depth Anything V2 pipeline and navigation. However, its current main stack targets ROS 1 Noetic, Isaac Sim 4.5 and CUDA 11.6, while a ROS 2 port is under development. Therefore it is best treated as a reference architecture and source of ideas/components rather than a direct drop-in for a new ROS 2 project. [R1]

## 4.2 Why the hybrid approach is preferable

Nav2 removes the need to implement a full navigation framework.

RTAB-Map removes the need to implement a complete visual SLAM system.

elevation_mapping_cupy removes much of the low-level elevation-map engineering.

Isaac Sim removes the need for a physical robot during the majority of development.

A small custom fusion layer can combine geometry and semantic terrain costs.

A small safety supervisor can enforce deterministic stop conditions.

# 5. Existing Software and Open-Source Landscape

| Project | Role | Fit | Important constraint |
| --- | --- | --- | --- |
| LARIAD/Offroad-Nav | Complete off-road reference stack | ★★★★★ | ROS 1 / older Isaac Sim baseline |
| ROS Navigation2 | Navigation framework | ★★★★★ | Needs robot-specific configuration |
| RTAB-Map ROS | Visual/RGB-D/stereo SLAM | ★★★★★ | Needs calibrated sensor/TF setup |
| elevation_mapping_cupy | Elevation + traversability | ★★★★★ | Sensor input and GPU compatibility must be validated |
| traversability-nav2 | Outdoor terrain costmap | ★★★★☆ | Reference implementation uses LiDAR-derived heightmaps |
| Ultralytics | Detection/segmentation/depth tasks | ★★★★☆ | Review licensing for intended deployment |
| Depth Anything V2 | Monocular depth | ★★★☆☆ | Small model has Apache-2.0; larger models have different terms |
| Open3D | 3D data processing | ★★★★☆ | General-purpose rather than full navigation stack |
| Clearpath simulator | Realistic UGV simulation option | ★★★★☆ | Gazebo ecosystem; hardware model differs |
| ORB-SLAM3 | Alternative visual/visual-inertial SLAM | ★★★★☆ | GPLv3 and older integration burden |

The project should choose one localization stack, not run RTAB-Map and ORB-SLAM3 simultaneously in the first version. Running two overlapping SLAM systems adds integration and debugging complexity without guaranteeing better navigation.

# 6. Technology Selection Matrix

| Layer | Primary choice | Fallback | Decision |
| --- | --- | --- | --- |
| Simulator | Isaac Sim 6.x | Gazebo Harmonic | Use Isaac Sim if GPU supports it |
| ROS | ROS 2 Jazzy | ROS 2 Humble | Prefer Jazzy for Ubuntu 24.04 |
| SLAM | RTAB-Map | ORB-SLAM3 | Use RTAB-Map first |
| Depth | Stereo/RGB-D | Depth Anything V2 | Prefer real metric depth |
| Detection | YOLO-family | Other detector | Use lightweight model initially |
| Segmentation | YOLO segmentation / dedicated segmentation | Classical segmentation | Start with only needed classes |
| Elevation | elevation_mapping_cupy | Custom grid map | Reuse mature GPU implementation |
| Traversability | Elevation + semantic cost fusion | traversability-nav2 | Custom thin fusion layer |
| Global planning | Nav2 planner | Custom A* | Nav2 |
| Local control | Nav2 MPPI | RPP/DWB | MPPI where compute allows |
| Debugging | RViz2 + Foxglove | rosbag2 only | Use both |
| Packaging | Docker/containers | Native installation | Containerize after baseline works |

# 7. Proposed Hybrid Architecture

┌───────────────────────┐
                         │      ISAAC SIM        │
                         │  UGV + terrain +      │
                         │  RGB / depth / IMU    │
                         └───────────┬───────────┘
                                     │ ROS 2
                 ┌───────────────────┼────────────────────┐
                 ▼                   ▼                    ▼
           RGB / Semantics       Depth/Cloud            IMU
                 │                   │                    │
                 ▼                   └──────────┬─────────┘
        ┌──────────────────┐                   │
        │ Vision Perception│                   │
        │ detection +      │                   │
        │ segmentation     │                   │
        └────────┬─────────┘                   │
                 │                             │
                 └──────────────┬──────────────┘
                                ▼
                    ┌─────────────────────────┐
                    │ Elevation / Traversable │
                    │ Map + Semantic Costs    │
                    └────────────┬────────────┘
                                 │
              ┌──────────────────┴──────────────────┐
              ▼                                     ▼
      ┌─────────────────┐                   ┌────────────────┐
      │    RTAB-MAP     │                   │     NAV2       │
      │ visual SLAM     │── pose ──────────►│ costmaps       │
      │ localization    │                   │ planner + MPPI │
      └─────────────────┘                   └───────┬────────┘
                                                   │
                                               /cmd_vel
                                                   │
                                                   ▼
                                         ┌──────────────────┐
                                         │ Safety Supervisor│
                                         │ can override     │
                                         └────────┬─────────┘
                                                  │
                                                  ▼
                                             SIMULATED UGV

The safety supervisor is deliberately outside the AI perception model. It monitors sensor freshness, localization health, obstacle distance, command validity and system state. It may reduce speed or block motion.

# 8. Detailed Perception System

## 8.1 Sensor strategy

A stereo camera or RGB-D camera is preferred over a single monocular camera because metric depth dramatically simplifies 3D geometry, elevation mapping and collision reasoning. If the competition strictly requires monocular input, Depth Anything V2 can be evaluated as an alternative depth source.

## 8.2 Perception outputs

| Output | Example | Consumer |
| --- | --- | --- |
| RGB frame | 1920×1080 or reduced resolution | Detector/segmenter |
| Depth map | meters per pixel | Point-cloud/elevation pipeline |
| Semantic mask | road/grass/mud/water/tree/person/rock | Traversability fusion |
| Obstacle detections | class + confidence + box/mask | Local costmap |
| Camera health | timestamp, frame age | Safety supervisor |
| Confidence | per-frame/model confidence | Speed policy |

## 8.3 Semantic classes

Traversable: compact dirt, road, grass when validated.

Caution: loose gravel, uneven grass, shallow rough terrain.

High cost: mud, water, deep vegetation, steep slope.

Lethal: ditch, cliff edge, large rock, tree trunk, wall.

Dynamic: human, vehicle, animal or other moving object.

## 8.4 Recommended first model

Use a lightweight YOLO-family model for object detection/segmentation and keep the initial class vocabulary small. Do not train a giant universal model. The model should answer the navigation questions that matter.

Engineering note: Ultralytics documentation currently supports detection, instance segmentation, semantic segmentation, depth estimation and related workflows, but its licensing terms must be reviewed for the intended competition and commercial use. [R9]

# 9. Visual Localization and SLAM

RTAB-Map is recommended as the first localization solution because its ROS 2 package provides examples for stereo and RGB-D cameras and includes robot/Nav2 integration examples. Its ROS 2 branch currently lists binaries/workflows for Humble, Jazzy and newer ROS distributions. [R4]

## 9.1 Data flow

Stereo/RGB-D + IMU
       │
       ▼
Camera calibration + TF
       │
       ▼
Visual odometry / feature matching
       │
       ▼
RTAB-Map graph
       │
       ├──► current pose
       ├──► map
       └──► loop closure
       │
       ▼
Nav2 localization / planning

## 9.2 Alternative: ORB-SLAM3

ORB-SLAM3 supports monocular, stereo, RGB-D and visual-inertial configurations and is a strong research-grade alternative. However, the repository is GPLv3, and its documented prerequisites/integration examples are older than the current ROS 2 stack. For this project it should be treated as a fallback/benchmark rather than the default path. [R8]

# 10. Elevation Mapping and Traversability

Outdoor navigation differs from indoor navigation because 'free space' is not binary. A surface can be geometrically open but physically unsafe. The system therefore needs a terrain representation.

Point cloud
    │
    ▼
Elevation Map
    │
    ├── height
    ├── variance
    ├── slope/gradient
    ├── roughness
    ├── visibility
    └── semantic layers
            │
            ▼
      Traversability cost
            │
            ▼
        Nav2 costmap

The elevation_mapping_cupy project is particularly relevant because it provides GPU-accelerated point-cloud registration/raycasting, height-drift compensation, visibility cleanup, semantic map support and learning-based traversability filtering. The repository has ROS 2 releases including Jazzy, making it a much better current baseline than a ROS 1-only elevation package. [R10]

The related traversability-nav2 project directly creates Nav2 traversability costmaps from terrain height/gradient information, but its reference pipeline is LiDAR-centric. The recommended adaptation is to feed it a camera-derived point cloud/elevation representation rather than rewriting the costmap concept. [R6]

# 11. Navigation, Planning and Control

Nav2 should be the navigation backbone. The current Nav2 repository contains planners, costmaps, behavior-tree navigation, collision monitoring, MPPI controller, regulated pure pursuit and related components, with active ROS 2 distribution support. [R3]

GOAL
                   │
                   ▼
             Global Planner
                   │
              global path
                   │
                   ▼
            Local Costmap
                   │
        ┌──────────┴──────────┐
        ▼                     ▼
  Static hazards        Dynamic hazards
        │                     │
        └──────────┬──────────┘
                   ▼
               MPPI Control
                   │
                   ▼
                /cmd_vel

## 11.1 Why MPPI

MPPI is attractive because it evaluates future control trajectories against the local environment rather than merely following a geometric line. This is useful for an outdoor UGV where turning radius, speed and obstacle proximity matter.

## 11.2 Why not write a custom A* + controller first?

Because the project goal is autonomous navigation, not implementation of textbook algorithms. The effort is better spent on terrain cost construction, sensor robustness, validation and integration.

# 12. Dynamic Obstacle Avoidance

Dynamic obstacle avoidance should be treated as a local replanning problem. The global route can remain valid while the local costmap changes.

t0:  START ─────────────────────► GOAL

t1:                    🚗
                       │
START ──────────────── X ───────► GOAL

t2:
START ────────────────┐
                       │
                       └──────────► GOAL

Detection → costmap update → local trajectory selection → continue/stop

Keep global planning relatively stable.

Update local obstacle costs rapidly.

Use the controller to reject trajectories entering lethal costs.

If the path is blocked, trigger a local replan.

If obstacle distance is below emergency threshold, bypass normal planning and stop.

# 13. Safety Supervisor and Fail-Safe Design

This is one of the most important custom modules. It should be small, deterministic and easy to audit.

if localization_lost:
    STOP

elif depth_stale:
    STOP

elif camera_stale:
    STOP

elif obstacle_distance < emergency_distance:
    STOP

elif perception_confidence < critical_confidence:
    SLOW_DOWN

elif path_is_invalid:
    STOP

else:
    ALLOW_NAV2_COMMAND

| Condition | Action | Reason |
| --- | --- | --- |
| No camera frames | STOP | Perception cannot be trusted |
| Depth timestamp stale | STOP | Obstacle distance may be invalid |
| Localization lost | STOP | Pose-dependent navigation unsafe |
| Lethal obstacle inside threshold | STOP | Avoid collision |
| Low semantic confidence | SLOW | Reduce risk while gathering information |
| No valid path | STOP/RECOVER | Do not drive blindly |
| Planner command invalid | STOP | Protect motor interface |

# 14. Simulation Strategy

Simulation is the main accelerator because the project can be developed before a physical UGV exists. The simulator must be treated as a test laboratory, not just a visualization tool.

## 14.1 Simulation levels

Teleoperate a virtual UGV.

Send a fixed goal through Nav2.

Navigate using simulated odometry.

Remove GPS and use visual SLAM.

Add depth and elevation mapping.

Add semantic perception.

Add dynamic obstacles.

Randomize lighting, terrain and obstacle placement.

Inject sensor failures.

Run hundreds/thousands of missions automatically.

## 14.2 Why simulation first

No hardware damage during failures.

Ground-truth pose is available for localization evaluation.

Exact obstacle locations are known.

Sensor noise can be controlled.

Weather/lighting can be randomized.

Experiments can be replayed.

A scenario can be repeated after every software change.

# 15. Isaac Sim Environment Design

NVIDIA Isaac Sim provides a ROS 2 bridge. Current NVIDIA documentation recommends ROS 2 Jazzy on Ubuntu 24.04, and also supports ROS 2 Humble/Jazzy on Ubuntu 22.04. The current Isaac Sim 6.0/6.0.1 requirements list a minimum x86_64 workstation configuration around 32 GB RAM, 50 GB SSD and a GeForce RTX 4080-class 16 GB VRAM GPU, with more resources recommended for advanced scenes and many sensors. [R11][R12]

## 15.1 Recommended virtual worlds

| World | Purpose | Examples |
| --- | --- | --- |
| Easy | Baseline navigation | Flat dirt, sparse rocks |
| Medium | Terrain reasoning | Grass, uneven ground, slopes, trees |
| Hard | Unstructured outdoor | Ditches, rocks, dense vegetation, narrow corridors |
| Dynamic | Collision avoidance | Moving humans/vehicles, suddenly appearing obstacles |
| Adversarial | Robustness | Strong shadows, glare, low light, sensor noise |
| Failure | Safety | Frozen camera, stale depth, SLAM loss, invalid commands |

## 15.2 Domain randomization

Sun angle and intensity.

Cloud cover and shadows.

Camera exposure/noise.

Terrain texture.

Obstacle size and location.

Starting pose and goal pose.

Vehicle speed.

Dynamic-object trajectories.

Depth noise and missing pixels.

# 16. ROS 2 Integration Architecture

/camera/rgb/image_raw
/camera/depth/image_rect_raw
/camera/camera_info
/imu/data
/odom
/tf
/tf_static
/rtabmap/odom
/map
/costmap
/plan
/cmd_vel
/safety/state
/safety/stop

Use ROS 2 interfaces as the stable contract. The simulated camera should publish the same semantic topic types and frame relationships that the real sensor driver will publish later. This minimizes the simulation-to-hardware rewrite.

## 16.1 TF tree

map
 └── odom
      └── base_link
           ├── camera_link
           │    ├── camera_left
           │    └── camera_right
           └── imu_link

Engineering note: A large fraction of robotics integration failures are frame, timestamp or calibration problems rather than AI problems. Establish and validate TF early.

# 17. Software Repository Map

| Ref | Repository | URL | Use |
| --- | --- | --- | --- |
| R1 | LARIAD/Offroad-Nav | https://github.com/LARIAD/Offroad-Nav | Closest end-to-end off-road reference; ROS 1 / Isaac Sim 4.5 baseline; MIT. |
| R2 | ros-navigation/navigation2 | https://github.com/ros-navigation/navigation2 | ROS 2 navigation framework; planners, costmaps, controllers, recovery and collision monitoring. |
| R3 | introlab/rtabmap_ros | https://github.com/introlab/rtabmap_ros | ROS 2 RTAB-Map package; stereo/RGB-D examples and Nav2 integration. |
| R4 | UZ-SLAMLab/ORB_SLAM3 | https://github.com/UZ-SLAMLab/ORB_SLAM3 | Visual, visual-inertial and multi-map SLAM; GPLv3. |
| R5 | leggedrobotics/elevation_mapping_cupy | https://github.com/leggedrobotics/elevation_mapping_cupy | GPU elevation mapping, semantic layers and traversability; MIT. |
| R6 | leggedrobotics/elevation_mapping_cupy_core | https://github.com/leggedrobotics/elevation_mapping_cupy_core | Standalone GPU elevation-map core; MIT. |
| R7 | sacrover/traversability-nav2 | https://github.com/sacrover/traversability-nav2 | Outdoor traversability Nav2 costmap plugin; LiDAR-oriented reference. |
| R8 | ultralytics/ultralytics | https://github.com/ultralytics/ultralytics | YOLO ecosystem: detection, segmentation, depth and deployment tooling; review license. |
| R9 | DepthAnything/Depth-Anything-V2 | https://github.com/DepthAnything/Depth-Anything-V2 | Monocular depth; small model Apache-2.0, larger variants have CC-BY-NC-4.0 terms. |
| R10 | isl-org/Open3D | https://github.com/isl-org/Open3D | 3D processing, registration, visualization and GPU acceleration. |
| R11 | isaac-sim/IsaacSim-ros_workspaces | https://github.com/isaac-sim/IsaacSim-ros_workspaces | ROS 2 workspaces for Isaac Sim; Apache-2.0. |
| R12 | clearpathrobotics/clearpath_simulator | https://github.com/clearpathrobotics/clearpath_simulator | Clearpath robot simulation with ROS 2 Jazzy + Gazebo Harmonic. |
| R13 | husky/husky | https://github.com/husky/husky | Clearpath Husky ROS packages; useful as a UGV model/reference. |

# 18. Data and AI Strategy

## 18.1 Start with pretrained models

Do not begin by collecting thousands of images. First establish the complete navigation loop with pretrained perception. Only collect/train a custom model after failure analysis shows that a pretrained model is the bottleneck.

## 18.2 Synthetic data

Isaac Sim can generate RGB, depth and other synthetic signals. A useful dataset should vary terrain, lighting, obstacle geometry and camera parameters. The generated data can be used for detector/segmenter fine-tuning and for stress testing.

## 18.3 Recommended model hierarchy

| Stage | Model | Purpose |
| --- | --- | --- |
| MVP | No learned segmentation | Prove ROS/SLAM/Nav2 pipeline |
| V1 | Lightweight YOLO detector | Objects: rock/tree/person/vehicle |
| V2 | YOLO segmentation or dedicated segmentation | Terrain classes |
| V3 | Stereo/RGB-D depth | Metric geometry |
| V4 optional | Depth Anything V2 Small | Monocular fallback/ablation |
| V5 | Custom fine-tuned model | Only if benchmark shows a clear gap |

# 19. Testing and Validation Plan

## 19.1 Test pyramid

┌────────────────────────────┐
                 │ Mission-level random tests │
                 │ 100–1000+ simulations      │
                 └──────────────┬─────────────┘
                                │
                     ┌──────────▼─────────┐
                     │ Scenario tests     │
                     │ terrain/obstacles  │
                     └──────────┬─────────┘
                                │
                     ┌──────────▼─────────┐
                     │ Component tests    │
                     │ SLAM/perception    │
                     └──────────┬─────────┘
                                │
                     ┌──────────▼─────────┐
                     │ Unit tests         │
                     │ safety/math/TF     │
                     └────────────────────┘

## 19.2 Core experiments

| Experiment | Variable | Measure |
| --- | --- | --- |
| Baseline navigation | Flat terrain | Goal success |
| Terrain | Slope/roughness | Collision + path efficiency |
| Perception | Lighting | Detection/segmentation degradation |
| Localization | Texture/feature density | ATE/RPE |
| Dynamic obstacle | Obstacle speed | Avoidance success |
| Sensor failure | Camera/depth loss | Time to stop |
| Planner stress | Narrow passages | Planning latency |
| Compute stress | Higher sensor FPS | CPU/GPU/latency |
| Randomized missions | All variables | Overall mission success |

## 19.3 Ground-truth advantage

Simulation gives exact ground-truth vehicle pose and object geometry. Store ground truth alongside ROS 2 bags so localization error, obstacle distance and path deviation can be computed objectively.

# 20. Evaluation Metrics

| Metric | Definition | Desired interpretation |
| --- | --- | --- |
| Mission success rate | Successful A→B missions / total missions | Primary outcome |
| Collision rate | Missions with any collision / total | Should approach zero |
| Near-miss rate | Minimum clearance below threshold | Safety sensitivity |
| Localization ATE | Absolute trajectory error | Visual localization quality |
| Localization RPE | Relative pose error | Short-term odometry quality |
| Path efficiency | Shortest feasible path / actual path | Planner quality |
| Planning latency | Time from map change to valid command | Responsiveness |
| Perception latency | Frame arrival to output | Real-time feasibility |
| Emergency-stop latency | Unsafe condition to blocked command | Safety quality |
| Compute utilization | GPU/CPU/RAM/VRAM | Deployment feasibility |
| Recovery success | Blocked mission recovered without collision | Robustness |

A single accuracy number is insufficient. A navigation system can have excellent object-detection mAP and still drive into a ditch. Mission-level metrics must therefore be the top-level score.

# 21. Machine and Hardware Requirements

## 21.1 Development workstation

Current NVIDIA Isaac Sim 6.0.1 documentation lists, for x86_64, a minimum configuration of Ubuntu 22.04/24.04 or Windows 11, 4 CPU cores, 32 GB RAM, 50 GB SSD, and a GeForce RTX 4080-class 16 GB VRAM GPU. NVIDIA's 'Good' configuration is 8 cores, 64 GB RAM, 500 GB SSD and an RTX 5080-class GPU; the 'Ideal' configuration is 16 cores, 64 GB RAM, 1 TB NVMe and an RTX PRO 6000 Blackwell-class GPU. NVIDIA notes that more RAM/VRAM is recommended for advanced scenes and that GPUs without RT cores such as A100/H100 are not supported for Isaac Sim. [R12]

| Tier | CPU | RAM | GPU | VRAM | Use case |
| --- | --- | --- | --- | --- | --- |
| Minimum | 4+ modern cores | 32 GB | RTX 4080-class | 16 GB | Basic Isaac Sim; limited sensors/scenes |
| Recommended | 8+ modern cores | 64 GB | RTX 5080-class | 16+ GB | Full project development |
| Comfortable | 12–16+ cores | 64–128 GB | High-end RTX / RTX PRO | 24–48 GB | Large worlds, many sensors, AI + simulation |
| Team server | 16+ cores | 128 GB+ | High-end RTX PRO | 48 GB+ | Parallel experiments / CI simulation |

## 21.2 Software requirements

Ubuntu 24.04 recommended for ROS 2 Jazzy + current Isaac Sim workflow.

NVIDIA GPU driver compatible with the selected Isaac Sim release.

ROS 2 Jazzy.

CUDA/toolkit versions matching the selected AI and elevation-mapping packages.

Python versions required by the selected ROS/Isaac Sim environment.

Git, CMake, colcon, rosdep.

Docker + NVIDIA Container Toolkit if containerized development is selected.

## 21.3 Future edge computer

For a physical UGV, a Jetson-class NVIDIA edge computer is a logical target, but the exact module should be selected only after profiling. The software should first be benchmarked on the development workstation, then optimized and measured on the intended edge device.

# 22. Installation and Development Plan

## 22.1 Recommended order

Install Ubuntu 24.04 and NVIDIA driver.

Install ROS 2 Jazzy.

Install Isaac Sim 6.x and validate with the NVIDIA Compatibility Checker.

Install/clone the Isaac Sim ROS 2 workspace.

Create a simple simulated UGV and camera.

Verify ROS 2 image/depth/TF topics.

Install Nav2 and make the virtual UGV drive to a goal.

Integrate RTAB-Map with stereo/RGB-D.

Add elevation_mapping_cupy.

Create the traversability cost layer.

Add lightweight perception.

Add semantic/geometry fusion.

Add safety supervisor.

Automate scenario generation and metrics.

## 22.2 Avoid dependency hell

Do not install every repository simultaneously.

Pin working versions once the baseline works.

Prefer official ROS packages when available.

Use one ROS distribution across the project.

Use containers for components with conflicting dependencies.

Keep each major subsystem independently testable.

# 23. Implementation Roadmap

| Phase | Duration | Deliverable | Do not add yet |
| --- | --- | --- | --- |
| 0 — Environment | 1–2 days | Ubuntu/ROS/Isaac Sim working | AI training |
| 1 — Sim navigation | 2–4 days | Virtual UGV reaches goal | SLAM |
| 2 — Visual SLAM | 3–6 days | GPS-free pose + map | Semantic AI |
| 3 — Terrain | 4–7 days | Elevation + traversability | Custom training |
| 4 — Perception | 5–10 days | Obstacle/terrain semantics | Complex multi-model fusion |
| 5 — Safety | 2–4 days | Deterministic stop/recovery | Hardware |
| 6 — Random testing | 5–10 days | Hundreds of missions + metrics | Real UGV |
| 7 — Optimization | 3–7 days | Latency/VRAM/FPS tuning | Feature creep |
| 8 — Hardware transfer | After simulation gate | Real sensor integration | Unvalidated driving |

The fastest path is to make each phase produce a runnable artifact. If Phase 1 does not work, do not proceed to Phase 4. This prevents a sophisticated AI model from hiding a broken TF, odometry or navigation foundation.

# 24. Failure Modes and Mitigations

| Failure | Likely cause | Mitigation |
| --- | --- | --- |
| Robot spins or drives incorrectly | TF/base-frame error | Validate TF tree and wheel geometry |
| SLAM drifts | Poor features, blur, exposure | Stereo/IMU, better camera, speed limits |
| Terrain map floats | Pose/depth synchronization | Timestamp checks + calibration |
| Rock missed | Model confidence/occlusion | Depth + semantic fusion + safety margin |
| Ditch classified as ground | Semantic/geometry mismatch | Slope/height risk + conservative cost |
| Dynamic obstacle collision | Slow local update | Fast costmap + MPPI + emergency threshold |
| Simulation runs slowly | GPU/VRAM overloaded | Lower resolution, sensors, scene complexity |
| AI latency too high | Large model | Smaller model/TensorRT/profiling |
| Unknown terrain | Out-of-distribution perception | Increase cost + slow/stop |
| Camera failure | Hardware/data stream fault | Safety supervisor |
| Localization loss | No visual features | Stop + relocalization/recovery |
| Planner has no path | Terrain genuinely blocked | Recovery/stop; do not force motion |

# 25. Security, Licensing and Open-Source Compliance

Open-source reuse is technically attractive, but every repository and model must be reviewed before redistribution or commercial deployment. Code licenses, model weights, datasets and third-party assets can have different terms.

| Component | Observed license/terms | Action |
| --- | --- | --- |
| LARIAD/Offroad-Nav | MIT | Preserve notices; verify bundled dependencies |
| Nav2 | Open-source ROS ecosystem | Preserve notices and dependency licenses |
| RTAB-Map ROS | BSD-3-Clause | Preserve notices |
| ORB-SLAM3 | GPLv3 | Legal review before closed-source/commercial integration |
| elevation_mapping_cupy | MIT | Preserve notices |
| elevation_mapping_cupy_core | MIT | Preserve notices |
| Depth Anything V2 Small | Apache-2.0 | Check model-specific terms |
| Depth Anything V2 Base/Large/Giant | CC-BY-NC-4.0 per repository | Do not assume commercial use is permitted |
| Ultralytics | License varies by distribution/use | Obtain appropriate license for intended use |
| Isaac Sim | NVIDIA proprietary software / EULA | Review NVIDIA EULA and deployment rights |

Engineering note: This is an engineering license screen, not legal advice. BEL/commercial deployment should receive a formal legal review.

# 26. Deployment Path to a Physical UGV

SIMULATION
Isaac Sim → ROS 2 → RTAB-Map → Traversability → Nav2
                         │
                         ▼
                 100s/1000s tests
                         │
                         ▼
                 Hardware interface
                         │
                         ▼
REAL UGV
Stereo/RGB-D + IMU → ROS 2 drivers
                         │
                         └── same perception/SLAM/Nav2 stack

The key to reducing hardware integration effort is to preserve the same ROS topic and TF contracts. Only the source of the sensor messages and low-level motor interface should change.

## 26.1 Hardware bring-up sequence

Bench-test camera and IMU streams.

Verify calibration and timestamps.

Verify TF frames while the UGV is stationary.

Drive manually at very low speed.

Validate odometry.

Run visual SLAM without autonomous control.

Run perception and map generation while teleoperated.

Enable Nav2 at low speed with an external emergency stop.

Increase speed only after repeated collision-free tests.

# 27. Recommended Final Stack

OS
 └── Ubuntu 24.04

Middleware
 └── ROS 2 Jazzy

Simulation
 └── NVIDIA Isaac Sim 6.x

Sensors
 ├── Stereo/RGB-D camera
 └── IMU

Localization
 └── RTAB-Map ROS 2

Perception
 ├── Lightweight YOLO detection/segmentation
 └── Stereo/RGB-D depth
       └── optional Depth Anything V2 Small

Terrain
 ├── elevation_mapping_cupy
 ├── slope / roughness
 └── semantic terrain cost

Navigation
 └── Nav2
      ├── Global planner
      ├── Local costmap
      ├── MPPI controller
      └── Recovery / collision monitor

Safety
 └── Custom deterministic supervisor

Debugging
 ├── RViz2
 ├── Foxglove
 └── rosbag2

3D utilities
 └── Open3D / PCL as required

Deployment
 ├── Docker where useful
 └── NVIDIA edge computer after profiling

This stack is intentionally hybrid: it uses proven open-source robotics for the difficult infrastructure while reserving custom development for the perception-to-traversability bridge and safety logic.

# 28. Competition Demonstration Plan

## 28.1 5-minute demo sequence

Show the virtual/physical UGV and destination.

Show camera feed and semantic perception.

Show the live elevation/traversability map.

Demonstrate GPS-denied localization.

Set Point A and Point B.

Let Nav2 drive the UGV.

Introduce a sudden obstacle.

Show local replan/avoidance.

Introduce an unsafe/unknown condition.

Show automatic slow-down or emergency stop.

Display mission metrics.

## 28.2 What judges should see

The robot understands terrain rather than merely following a painted road.

The robot can localize visually without GPS.

The robot reacts to newly appearing obstacles.

The robot behaves conservatively when uncertain.

The architecture is modular and can move from simulation to real hardware.

The team can prove performance quantitatively.

# 29. Conclusion

The recommended solution is a simulation-first hybrid autonomy stack. The project should not attempt to build a complete navigation system from scratch. The strongest low-effort architecture is Isaac Sim + ROS 2 Jazzy + RTAB-Map + GPU elevation/traversability mapping + Nav2/MPPI, with lightweight vision AI and a deterministic safety supervisor.

The closest existing end-to-end reference found is LARIAD/Offroad-Nav, but it currently represents a ROS 1/Isaac Sim 4.5 baseline. The modern implementation should reuse the architectural ideas while moving the navigation layer to current ROS 2 tooling. RTAB-Map provides a practical visual localization path; elevation_mapping_cupy provides a strong terrain-map foundation; and Nav2 provides the navigation/control infrastructure.

The engineering objective should be stated as: 'Demonstrate repeatable collision-free GPS-denied outdoor navigation across a controlled, randomized test suite, with quantitative localization, perception, planning, latency and safety metrics.' That is a technically defensible target and a much stronger competition story than claiming generic 100% AI accuracy.

# Appendix A — Repository and Software Reference

[RR1] LARIAD/Offroad-Nav — https://github.com/LARIAD/Offroad-Nav — Closest end-to-end off-road reference; ROS 1 / Isaac Sim 4.5 baseline; MIT.

[RR2] ros-navigation/navigation2 — https://github.com/ros-navigation/navigation2 — ROS 2 navigation framework; planners, costmaps, controllers, recovery and collision monitoring.

[RR3] introlab/rtabmap_ros — https://github.com/introlab/rtabmap_ros — ROS 2 RTAB-Map package; stereo/RGB-D examples and Nav2 integration.

[RR4] UZ-SLAMLab/ORB_SLAM3 — https://github.com/UZ-SLAMLab/ORB_SLAM3 — Visual, visual-inertial and multi-map SLAM; GPLv3.

[RR5] leggedrobotics/elevation_mapping_cupy — https://github.com/leggedrobotics/elevation_mapping_cupy — GPU elevation mapping, semantic layers and traversability; MIT.

[RR6] leggedrobotics/elevation_mapping_cupy_core — https://github.com/leggedrobotics/elevation_mapping_cupy_core — Standalone GPU elevation-map core; MIT.

[RR7] sacrover/traversability-nav2 — https://github.com/sacrover/traversability-nav2 — Outdoor traversability Nav2 costmap plugin; LiDAR-oriented reference.

[RR8] ultralytics/ultralytics — https://github.com/ultralytics/ultralytics — YOLO ecosystem: detection, segmentation, depth and deployment tooling; review license.

[RR9] DepthAnything/Depth-Anything-V2 — https://github.com/DepthAnything/Depth-Anything-V2 — Monocular depth; small model Apache-2.0, larger variants have CC-BY-NC-4.0 terms.

[RR10] isl-org/Open3D — https://github.com/isl-org/Open3D — 3D processing, registration, visualization and GPU acceleration.

[RR11] isaac-sim/IsaacSim-ros_workspaces — https://github.com/isaac-sim/IsaacSim-ros_workspaces — ROS 2 workspaces for Isaac Sim; Apache-2.0.

[RR12] clearpathrobotics/clearpath_simulator — https://github.com/clearpathrobotics/clearpath_simulator — Clearpath robot simulation with ROS 2 Jazzy + Gazebo Harmonic.

[RR13] husky/husky — https://github.com/husky/husky — Clearpath Husky ROS packages; useful as a UGV model/reference.

# Appendix B — Example ROS 2 Topic Contract

| Topic | Type / semantic role | Publisher | Consumer |
| --- | --- | --- | --- |
| /camera/rgb/image_raw | RGB image | Isaac Sim / camera driver | Perception |
| /camera/depth/image_rect_raw | Metric depth | Isaac Sim / camera driver | Elevation/obstacle pipeline |
| /camera/camera_info | Calibration | Camera driver | Perception/SLAM |
| /imu/data | IMU | Isaac Sim / IMU driver | SLAM/odometry |
| /odom | Odometry | Simulator/odometry | SLAM/Nav2 |
| /tf | Transforms | TF broadcasters | All |
| /map | Map | SLAM | Nav2/RViz |
| /rtabmap/odom | Visual odometry | RTAB-Map | Nav2/diagnostics |
| /global_costmap/costmap | Global costs | Nav2 | Planner |
| /local_costmap/costmap | Local costs | Nav2 + custom layers | Controller |
| /plan | Planned path | Nav2 | Controller/RViz |
| /cmd_vel | Velocity command | Nav2/safety supervisor | UGV base |
| /safety/state | Safety state | Custom supervisor | Operator/dashboard |

# Appendix C — Example Cost Function

A useful traversability cost can combine geometric and semantic terms. One conceptual formulation is:

C(x) =
    w_s * slope_cost(x)
  + w_r * roughness_cost(x)
  + w_h * height_variance(x)
  + w_o * obstacle_cost(x)
  + w_m * semantic_cost(x)
  + w_u * uncertainty_cost(x)

The exact weights should be learned through controlled experiments. The safety philosophy should make unknown terrain expensive rather than free.

# Appendix D — Test Scenario Catalog

| ID | Scenario | Purpose | Severity |
| --- | --- | --- | --- |
| T01 | Flat open dirt | Baseline | Low |
| T02 | Sparse rocks | Obstacle detection | Low |
| T03 | Dense rocks | Obstacle avoidance | Medium |
| T04 | Tree corridor | Semantic obstacles | Medium |
| T05 | Positive slope | Terrain geometry | Medium |
| T06 | Negative slope | Terrain geometry | Medium |
| T07 | Ditch crossing | Safety/traversability | High |
| T08 | Mud patch | Semantic terrain | High |
| T09 | Water-like surface | Semantic + geometry | High |
| T10 | Low-light | Vision robustness | High |
| T11 | Strong sunlight/glare | Vision robustness | High |
| T12 | Moving pedestrian | Dynamic avoidance | High |
| T13 | Moving vehicle | Dynamic avoidance | High |
| T14 | Sudden obstacle | Reaction latency | High |
| T15 | Featureless terrain | SLAM robustness | High |
| T16 | Camera dropout | Safety | Critical |
| T17 | Depth dropout | Safety | Critical |
| T18 | SLAM loss | Safety/recovery | Critical |
| T19 | No valid path | Planner recovery | Critical |
| T20 | Randomized mission | End-to-end | Critical |

# Appendix E — References

[RR1] LARIAD Offroad-Nav — https://github.com/LARIAD/Offroad-Nav — Open-source LiDAR/monocular off-road UGV navigation reference. Repository states ROS 1 Noetic, Isaac Sim 4.5 and CUDA 11.6 for its current main setup.

[RR2] ROS Navigation2 — https://github.com/ros-navigation/navigation2 — ROS 2 Navigation Framework and System.

[RR3] RTAB-Map ROS 2 — https://github.com/introlab/rtabmap_ros — ROS 2 package with stereo/RGB-D examples and Nav2 integration.

[RR4] ORB-SLAM3 — https://github.com/UZ-SLAMLab/ORB_SLAM3 — Visual, visual-inertial and multi-map SLAM library.

[RR5] Nav2 Traversability Plugin — https://github.com/sacrover/traversability-nav2 — Traversability costmap plugin for outdoor terrain.

[RR6] Elevation Mapping CuPy — https://github.com/leggedrobotics/elevation_mapping_cupy — GPU-accelerated elevation mapping and traversability/semantic layers.

[RR7] Elevation Mapping CuPy Core — https://github.com/leggedrobotics/elevation_mapping_cupy_core — Standalone ROS-free GPU elevation map core.

[RR8] Open3D — https://github.com/isl-org/Open3D — 3D data processing and visualization library.

[RR9] Ultralytics — https://github.com/ultralytics/ultralytics — YOLO detection/segmentation/depth ecosystem.

[RR10] Depth Anything V2 — https://github.com/DepthAnything/Depth-Anything-V2 — Monocular depth estimation model family.

[RR11] Isaac Sim ROS Workspaces — https://github.com/isaac-sim/IsaacSim-ros_workspaces — ROS 2 workspaces for Isaac Sim.

[RR12] Isaac Sim ROS 2 Installation — https://docs.isaacsim.omniverse.nvidia.com/latest/installation/install_ros.html — NVIDIA ROS 2 bridge and supported ROS distributions.

[RR13] Isaac Sim 6.0.1 Requirements — https://docs.isaacsim.omniverse.nvidia.com/6.0.1/installation/requirements.html — Current workstation requirements and compatibility notes.

[RR14] Clearpath Simulator — https://github.com/clearpathrobotics/clearpath_simulator — ROS 2 Jazzy + Gazebo Harmonic simulation for Clearpath robots.

[RR15] Clearpath Husky packages — https://github.com/husky/husky — UGV simulation/control/description reference.

Research note: URLs and software characteristics were checked against public sources on 4 September 2026. Software releases, compatibility and license terms can change; pin versions and re-check official documentation before deployment.